SURVEYOR’S REPORT

Hydrographic Examination Survey

USCG Station Dulac

Dulac, Terrebonne Parish, Louisiana

Report of Survey:  Andrew Oakman, U.S. Army Corps of Engineers, New Orleans District

Project: USCG Station New Orleans, Examination Survey, FY25

Location:  241 Coast Guard Rd, Dulac 70353, Terrebonne Parish, Louisiana.

Date:  Field data was acquired on 29 September 2025.  Additional occupation of Benchmark GC-3 acquired on 03 October 2025.

Right of Access: Proper identification must be shown for access into the USCG Base. Access to the USCG Station Dulac is through a security gate at the end of Coast Guard Road. POC for the Station Dulac is: Officer of the Day, 409-988-1257. The boat ramp used for launching and loading the survey boat is on the east side of the property.

Personnel:  System Operator/Boat Captain: Andrew Oakman

Datum: The horizontal coordinates are referenced to the State Plane Coordinate System, Louisiana South, North American Datum of 1983 (NAD 83).  Soundings are referenced to Mean Lower Low Water (MLLW) from the latest tidal epoch (1983-2001).

Instruments: The following instruments were used for data collection:

| ECHOBOAT 160 |  |
| --- | --- |
| Data Collection Software | HYPACK 2025 |
| Multibeam Sounder | NORBIT SeaRay |
| Positioning | Applanix POS MV |

Field Procedures:  This survey was performed utilizing Real-Time Kinematic Network (RTN) GPS.  All tidal reductions were obtained utilizing RTK and referenced to MLLW using *single point datum relationship between NAVD and MLLW using VDatum’s Online Conversion Tool.  Sound velocity determined from direct sound velocity measurement at sonar head. * GC-3 has been disturbed since 2022, concrete broken and probable translation occurred.

| Monument | PID | X Coordinate (sft) | Y Coordinate (sft) | Geoid Model | Elevation (sft) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | NAVD88 | MLLW |
| GC 3* | AH6332 | 3478412.06 | 319555.23 | 18 | 2.74 | 3.18 |
| TBM 1 SHEARED CONDUIT |  | 3477975.81 | 319693.53 | 18 | 4.38 | 4.82 |
| TBM 2 N. BIT BOLT |  | 3477993.71 | 319658.73 | 18 | 4.83 | 5.27 |
| USCG BM |  | 3477913.76 | 319629.49 | 18 | 3.17 | 3.61 |
| STAFF GAGE 4FT MARK |  | 3478035.44 | 319698.68 | 18 | 3.61 | 4.05 |

Data Collection:  A multibeam system was utilized for entire project. All mapping done in 200 kHz frequency.

Data Processing:  The survey data was processed upon completion of the survey.  Sounding spikes were removed and cross line statistics were performed for each day of survey with no anomalies found.

Field Conditions: Mostly calm. Heavy marine traffic throwing wake.

OVERVIEW MAP NOTES:

1. REFER TO XYZ WITH SURVEY NAME: HN_00_CGD_20250929_CS_05X05_MLLW_A.XYZ

2. DEPTHS DEPICTED BY THIS SURVEY ARE REFERENCED TO MEAN LOWER LOW WATER (MLLW), TIDAL EPOCH 1983-2001, AS REPORTED BY THE NATIONAL OCEANIC AND ATMOSPHERIC ADMINISTRATION (NOAA). NOAA’S VDATUM MODEL ONLINE CONVERSION TOOL WAS USED WITH A SINGLE POINT NAVD TO MLLW AT LAT 29.202801  LON -90.665423 FOR THE ENTIRE PROJECT.

0.0’ NAVD88 (2009.55) = 0.44’ MLLW

3. TIDAL REDUCTIONS WERE OBTAINED UTILIZING LSU C4GNET REAL-TIME KINEMATIC GPS AND REFERENCED TO NAVD88 (2009.55) WATER SURFACE ELEVATIONS, NOT CALIBRATED TO MONUMENT GC 3.  ELEVATIONS WERE CORRECTED TO MLLW USING THE RELATIONSHIP IN NOTE 2.

4. ALL ELEVATIONS ARE BELOW THE CHART DATUM UNLESS PRECEEDED BY A (+) SIGN.

5. PLANE COORDINATES ARE BASED ON THE LAMBERT CONFORMAL CONIC PROJECTION FOR THE SOUTH ZONE OF LOUISIANA AND REFERENCED TO THE NORTH AMERICAN DATUM OF 1983 (NAD 83).

6. VERTICAL MEASUREMENTS WERE MADE USING A NORBIT SEARAY MULTIBEAM SYSTEM.

SURVEY VESSEL                    DATE OF SURVEY                                                         AREA

ECHOBOAT 160	         29 SEP/03 OCT 2025	                    	USCG STA DULAC

8. THE INFORMATION DEPICTED ON THIS MAP REPRESENTS THE RESULTS OF SURVEYS MADE ON THE DATES INDICATED ABOVE AND CAN ONLY BE CONSIDERED AS INDICATING THE GENERAL CONDITIONS AT THAT TIME.  THIS CHART IS SOLELY FOR THE DISTRIBUTION OF AVAILABLE DEPTHS AT THE TIME OF SURVEY.

9. SURVEY ACCURACY STANDARDS, QUALITY CONTROL, AND QUALITY ASSURANCE REQUIREMENTS WERE FOLLOWED DURING THIS SURVEY IN ACCORDANCE WITH USACE EM 1110-2-1003, HYDROGRAPHIC SURVEYING, 30 NOV 13.

10. VOLUME OF MATERIAL TO BE REMOVED EQUALS 1,530 CUBIC YARDS GIVEN THE PROVIDED TOE LINES AT -11’ MLLW.

Appendix A

GC 3

TBM 1 :  SHEARED CONDUIT

TBM 2 :  N. BIT BOLT

USCG BM

STAFF GAGE ON WHARF